Reverse Lane

Detailed Runtime, Object and Diagram Documentation

Recommended reading order: overview -> architecture -> runtime flow -> gameplay logic -> object details -> build/flash.

# 1. Overview

Reverse Lane is a compact 1-bit OLED lane-dodge game for the AK Embedded Base Kit STM32L151. The player controls a vehicle moving against traffic. Incoming vehicles move from right to left across four lanes. The player survives by changing lanes, jumping over objects and avoiding collisions as speed and spawn pressure increase.

Game type: lane dodge / reverse traffic survival.

Display: 128 x 64 monochrome OLED.

Core firmware style: AK event-driven message and timer architecture.

Main gameplay file: application/sources/app/screens/scr_game.cpp.

# 2. Hardware Target

| Item | Project value |
| --- | --- |
| MCU | STM32L151CBT6 / Arm Cortex-M3 |
| Display | 128 x 64 monochrome OLED |
| Input | SW3 = Up, SW2 = Down, SW4 = Mode / Jump / Restart |
| Boot address | 0x08000000 |
| Application address | 0x08003000 |

# 3. Architecture Block Flow

This is the highest-level view of the firmware path. Hardware input and timer events both become AK messages. The display task dispatches those messages into the active screen. The screen updates state, then the screen manager asks the render layer to refresh OLED.

Button / Timer -> AK Message Queue -> task_display()
task_display() -> scr_mng_dispatch(msg)
screen_manager -> current screen handler
screen handler -> state update
screen_manager -> view_render_screen(view_screen) -> OLED

# 4. Runtime / Thread-Time Flow

In this project, “thread-time” means logical event timelines, not operating-system threads. Each timeline posts messages into AC_TASK_DISPLAY_ID. The current screen determines what the signal means.

| Time Thread | Signal/Event | Producer | Consumer | Meaning |
| --- | --- | --- | --- | --- |
| Power-up thread | AC_DISPLAY_INITIAL | app.cpp timer after application init | scr_startup_handle() | Initialize OLED and show logo. |
| Logo delay thread | AC_DISPLAY_SHOW_LOGO after 2000 ms | timer_set in startup screen | scr_startup_handle() | Hide logo and reveal menu. |
| Button thread | UP/DOWN/MODE pressed | app_bsp.cpp button callbacks | current screen via task_display | Move menu/game player or trigger jump/restart/back. |
| Game tick thread | AC_DISPLAY_GAME_TICK every 80 ms | timer_set in scr_game_handle(SCREEN_ENTRY) | scr_game_handle() | Run one frame of gameplay. |
| Render throttle thread | AC_DISPLAY_RENDER_SCREEN one-shot when needed | screen_manager.cpp | screen_manager | Prevent rendering faster than 50 ms. |

## 4.1 Startup Logo and Menu Sequence

| Time | Where | Action | Result |
| --- | --- | --- | --- |
| t=0 | app.cpp | Tasks, buttons and screen manager are initialized. | scr_startup is current screen. |
| t+100 ms | AC_DISPLAY_INITIAL | OLED is initialized and display is turned on. | Logo bitmap is drawn. |
| t+100 ms | scr_startup_handle() | timer_set(AC_DISPLAY_SHOW_LOGO, 2000 ms, ONE_SHOT). | Logo delay starts. |
| t+2 s | AC_DISPLAY_SHOW_LOGO | startup_show_logo becomes false. | Menu appears. |
| SW4 short | AC_DISPLAY_BUTON_MODE_PRESSED | If menu index is 0, call SCREEN_TRAN(scr_game_handle, &scr_game). | Game receives SCREEN_ENTRY. |

## 4.2 Game Entry Sequence

| Step | Code | Effect |
| --- | --- | --- |
| 1 | SCREEN_TRAN(scr_game_handle, &scr_game) | Screen manager changes current screen and sends SCREEN_ENTRY. |
| 2 | scr_game_handle(SCREEN_ENTRY) | Calls game_init() to reset score, level, player, jump state and vehicle slots. |
| 3 | timer_set(... AC_DISPLAY_GAME_TICK, 80, TIMER_PERIODIC) | Creates periodic game frame event every 80 ms. |
| 4 | scr_mng_dispatch() | After each signal, screen manager renders if the 50 ms interval allows it. |

# 5. Screen Flow

| Screen | Handler | Render callback | Responsibility |
| --- | --- | --- | --- |
| Startup | scr_startup_handle() | view_scr_startup() | Logo, Vietnamese menu, transition into game. |
| Game | scr_game_handle() | view_scr_game() | Timer tick, buttons, gameplay state, render lanes and objects. |
| Idle/Welcome/QR | existing project screens | their dynamic views | Legacy sample screens from the original kit project. |

# 6. Button Message Flow

Buttons are translated into messages in app_bsp.cpp. The game screen does not directly read GPIO for normal up/down/jump events. It receives display signals from the AK message queue.

| Physical button | Callback | Posted signal | Startup meaning | Game meaning |
| --- | --- | --- | --- | --- |
| SW3 / UP | btn_up_callback() | AC_DISPLAY_BUTON_UP_PRESSED | Menu up or skip logo. | Move player up; restart after game over. |
| SW2 / DOWN | btn_down_callback() | AC_DISPLAY_BUTON_DOWN_PRESSED | Menu down or skip logo. | Move player down; restart after game over. |
| SW4 / MODE short | btn_mode_callback() | AC_DISPLAY_BUTON_MODE_PRESSED | Select or skip logo. | Jump / return / restart. |
| SW4 / MODE long | btn_mode_callback() | AC_DISPLAY_BUTON_MODE_LONG_PRESSED | Unused in menu. | Stop game tick and return to startup. |

# 7. Game Tick Flow

The 80 ms game tick is the main gameplay heartbeat. Each tick runs one deterministic update cycle, then rendering happens through the screen manager.

| Order | Function/Block | State touched | Why it matters |
| --- | --- | --- | --- |
| 1 | game_update_difficulty() | score, level, spawn_period | Maps score to level 1/2/3/4 and changes spawn interval. |
| 2 | SW2+SW3 hold return check | player_returning, player_x | Allows controlled return toward base x. |
| 3 | Jump forward phase | player_jump_ticks, player_x | Moves player forward during jump; level >= 3 adds speed. |
| 4 | Jump countdown / return | jump/return flags | Controls airborne, returning or delayed jump states. |
| 5 | Spawn check | spawn_counter, vehicles[] | Creates new incoming vehicles when counter reaches zero. |
| 6 | Vehicle update loop | vehicles[i].x/lane/scored | Lane change, movement, scoring and off-screen deactivation. |
| 7 | Collision check | player/vehicle hitboxes | Ends game unless jump rules clear the overlap. |
| 8 | Render after dispatch | OLED framebuffer | Draws HUD, lanes, vehicles, player and game over overlay. |

# 8. Gameplay Logic

## 8.1 Difficulty and Score

| Level | Score range | Spawn period base | Speed bonus | Behavior |
| --- | --- | --- | --- | --- |
| 1 | 0 - 9 | 28 ticks | +1 px/tick | Basic dodge and jump learning. |
| 2 | 10 - 49 | 23 ticks | +2 px/tick | Lane-changing vehicles become relevant. |
| 3 | 50 - 99 | 18 ticks | +3 px/tick | Higher speed can clear larger objects during jump. |
| 4 | 100+ | 13 ticks, clamped >= 11 | +4 px/tick | Highest pressure and chance for extra spawn. |

## 8.2 Collision and Jump Logic

| Condition | Result |
| --- | --- |
| Player is not jumping and hitboxes overlap in same lane. | Game over. |
| Player is jumping and object is Moto/F1. | Collision is skipped because object is jumpable. |
| Player is jumping and object is Container. | Allowed only if jump_has_clear_speed() returns true. |
| Jump phase is before 3 or after 11. | Too early/late for large-object clearance. |
| Level >= 3 or player_x >= PLAYER_START_X + 16. | Enough speed/position to clear large object during useful jump window. |

Collision summary:
same lane -> x hitbox overlap -> if airborne check jumpable/speed -> else game_over

# 9. Object Specification

Objects are placed after the runtime/gameplay flow because they are easier to understand once the reader knows where spawning, movement, collision and rendering happen.

| Object | Declared in | Bitmap size | Runtime type |
| --- | --- | --- | --- |
| Player | bitmap_player[] in scr_game.cpp | 17 x 18 px | Player-only sprite, not stored in vehicle_types[] |
| Jump Player | bitmap_player_large[] in scr_game.cpp | 24 x 17 px | Temporary airborne player sprite |
| Moto | bitmap_moto[] + vehicle_types[0] | 18 x 10 px | Incoming vehicle object |
| F1 | bitmap_f1[] + vehicle_types[1] | 27 x 10 px | Incoming vehicle object |
| Container | bitmap_container[] + vehicle_types[2] | 40 x 13 px | Incoming vehicle object |

## 9.1 Bitmap Storage Format

All gameplay objects are stored as 1-bit monochrome C byte arrays in PROGMEM. Each bit represents one OLED pixel. The render layer draws them using view_render.drawBitmap().

static const unsigned char PROGMEM bitmap_player[] = { ... };
view_render.drawBitmap(x, y, bitmap_player, PLAYER_WIDTH, PLAYER_HEIGHT, WHITE);

Bitmap width/height is visual size, not always collision size.

Collision uses smaller hitboxes to avoid unfair deaths.

Vehicles spawn at the right edge and move left by step + level_speed_bonus() per tick.

## 9.2 Player Object

| Property | Value / Logic | Explanation |
| --- | --- | --- |
| Bitmap | bitmap_player[] | Normal player vehicle sprite. |
| Visual size | 17 x 18 px | Centered on selected lane. |
| Start position | PLAYER_START_X = 6, player_lane = 1 | Starts near left side in the second lane. |
| Lane movement | UP -> lane--, DOWN -> lane++ | Allowed only when player_jump_ticks == 0. |
| Hitbox | player_x + 4, width 11 px | Smaller than bitmap width for fair collision. |
| Render y | lane_center_y(player_lane) - PLAYER_HEIGHT / 2 + jump_y | Centers player and applies jump offset. |

## 9.3 Jump Player Object

| Property | Value / Logic | Explanation |
| --- | --- | --- |
| Bitmap | bitmap_player_large[] | Airborne sprite. |
| Visual size | 24 x 17 px | Looks larger while jumping. |
| Duration | 16 ticks = about 1.28 s | 80 ms per tick. |
| Vertical path | jump_y[] | Negative y moves sprite upward. |
| Forward path | jump_auto_dx[] | Moves forward during jump; level >= 3 adds +1 px/tick. |
| Clear window | phase 3..11 | Useful window for clearing large objects. |

## 9.4 Moto Object

| Property | Value / Logic | Explanation |
| --- | --- | --- |
| Bitmap | bitmap_moto[] | Small incoming vehicle sprite. |
| Visual size | 18 x 10 px | Smallest enemy object. |
| Base step | 2 px/tick | Actual movement is 2 + level bonus. |
| Hitbox | hit_x = 3, hit_w = 12 | Central danger area only. |
| Jumpable | true | Airborne player can clear it. |
| Lane change | true from level >= 2 | May change lane if nearby lane is safe. |

## 9.5 F1 Object

| Property | Value / Logic | Explanation |
| --- | --- | --- |
| Bitmap | bitmap_f1[] | Race car / F1 incoming object. |
| Visual size | 27 x 10 px | Longer than moto, still lane-friendly. |
| Base step | 3 px/tick | Fastest base object. |
| Hitbox | hit_x = 4, hit_w = 19 | Ignores transparent edge pixels. |
| Jumpable | true | Can be cleared during jump. |
| Lane change | true from level >= 2 | Adds unpredictability in mid/late game. |

## 9.6 Container Object

| Property | Value / Logic | Explanation |
| --- | --- | --- |
| Bitmap | bitmap_container[] | Large truck/container object. |
| Visual size | 40 x 13 px | Longest object. |
| Base step | 1 px/tick | Slow base speed, but dangerous as level bonus rises. |
| Hitbox | hit_x = 4, hit_w = 32 | Large central danger area. |
| Jumpable | false by default | Requires jump_has_clear_speed() to clear. |
| Lane change | false | Stays in spawn lane for readability/fairness. |

## 9.7 Spawn, Lane Change and Hitbox Rules

| Rule | Code behavior | Reason |
| --- | --- | --- |
| Pool slot | Find first inactive vehicles[slot]. | Avoid dynamic memory during gameplay. |
| Lane choose | rand() % ROAD_LANE_COUNT | Random one of four lanes. |
| Spawn gap | lane_has_near_vehicle(..., 44) | Avoid unfair same-lane stacking. |
| Default object | rand() % 2 | Early game mostly moto/F1. |
| Container introduction | level >= 2 probability | Add long object after score 10. |
| Lane change object | Moto/F1 only, level >= 2 | Moving threat without making container unfair. |
| Collision model | same lane + x hitbox overlap | Simple and cheap for MCU. |

# 10. Build / Flash

Build boot only when bootloader/startup boot code changes: run make inside boot/.

Build application when changing menu, gameplay, bitmap, input or render: run make inside application/.

Flash boot binary to 0x08000000.

Flash application binary to 0x08003000.

For gameplay-only changes, flash application only. Avoid full chip erase unless restoring both boot and application.

# 11. Source File Index

| Question during presentation | Best file to open | What to show |
| --- | --- | --- |
| Where is game logic? | scr_game.cpp | game_update(), game_spawn_vehicle(), game_check_collision(), view_scr_game(). |
| Where are objects? | scr_game.cpp | bitmap arrays and vehicle_types[]. |
| How do buttons reach game? | app_bsp.cpp | button callbacks posting AC_DISPLAY_* messages. |
| How is screen changed? | screen_manager.cpp | scr_mng_tran() and SCREEN_ENTRY. |
| How is rendering triggered? | screen_manager.cpp + view_render.cpp | scr_mng_dispatch() and render throttle. |
| How is periodic update created? | scr_game.cpp | timer_set(... AC_DISPLAY_GAME_TICK, 80, TIMER_PERIODIC). |

# 12. Short Presentation Script

Reverse Lane is an event-driven OLED game. The board starts in the startup screen, initializes the OLED, shows a logo, then displays a Vietnamese menu. Selecting Reverse Lane transitions into the game screen. On SCREEN_ENTRY, the game resets state and starts an 80 ms periodic timer. Each timer message runs game_update(): difficulty, player jump/return state, vehicle spawn, movement, scoring and collision. Buttons are converted to AK messages in app_bsp.cpp, so the active screen decides what each button means. Rendering is separated through screen_manager and view_render, which keeps the gameplay logic clean and close to the AK Embedded project style.